# CL5000 Short Guide

- How to Update Firmware

Rev. : 2007. 2. 27

< CL5000 Update File Structure>

* X_CODE0-[Date][Version(Checksum)].Bin	: Firmware binary file

* CLP-MA-[Country]-[Version].Rom		: Data ROM file (caption, font, parameter etc)

* CL5fwupdate.exe				: When downloading firmware and data rom file

< Firmware & ROM Update Method >

0. If necessary, you can do to backup scale memory.

- PLU, KeyMap, Label

- Firmware Update - Using "CL5FWupdate.exe" program, you can update firmware

ex) "X_CODE0-20061221 V13000S558(C110).BIN" file

(Notice - Please, check your connected port, firmware ROM and key type.)

Step 1. Press  , ready to F/W download

Step 2. Turn off and on the scale to get scale ready

Step 3. Finish the firmware download

2. Scale Memory Clear

- Using "Memory Clear" Button

3. After about 3 second, you should turn off and on.

- As turnning on, memory initialize autometically.

- You should wait for a few time till finishing Memory Initialization.

4. After finishing memory initialization, you can appear "rdy nEED CAP" message on the VFD Display.

- During Memory Initialization If "param" message appears and stays on VFD Display, turn off the scale and turn on the scale.

- If turning on after turning off the scale it displays "rdy nEED CAP" message.

5. Data ROM Update - Use "CL5FWupdate.exe" to download "CLP-MA-[COUNTRY]-01V22.rom"

- Press  , ready to ROM download